Desert station

This dataset includes data recorded by the automatic weather station (AWS) from 2015/05/02 to 2021/12/31. The Desert station (100.99° E, 42.11° N) was covered by Reaumuria and located in Ejin Banner, Inner Mongolia autonomous region. The elevation is 1054m.

An overview of the measured variables and sensor specifications are summarized in the following Table:

| Variable | Unit | Corporation/Sensors | Height | Direction |
| --- | --- | --- | --- | --- |
| Air temperature | ℃ | Vaisala/HMP45AC | 5,10 m | north |
| Air humidity | % | Vaisala/HMP45AC | 5,10 m | north |
| Wind speed | m/s | Met One/010C | 5,10 m | north |
| Wind direction | ° | Met One/020C | 5,10 m | north |
| Air pressure | hpa | Campbell/CS100 | 2 m | / |
| Precipitation | mm | Texas Electronics/ TE525MM | 10 m | / |
| Four-component radiation ((DR/UR/DLR_Cor/ULR_Cor) | W/m2 | Kipp&Zone/CNR1 | 6 m | south |
| Infrared temperature | ℃ | Apogee/SI-111 | 6 m | south, vertically downward |
| Soil heat flux | W/m2 | Hukseflux/HFP01 | -6 cm (3 Repeats) | / |
| Soil temperature | ℃ | Delta-T/ML3 | 0, -2, -4, -10, -20, -40, -60, and -100 cm | / |
| Soil moisture | % | Delta-T/ML3 | -2, -4, -10, -20, -40, -60, and -100 cm | / |

The observed items include: air temperature and humidity (Ta_5m, RH_5m, Ta_10m, RH_10m), pressure(Press), precipitation (Rain), wind speed (WS_5m, WS_10m), wind direction (WD_10m), four-component radiation (DR, UR, DLR_Cor, ULR_Cor, Rn), infrared temperature (IRT_1, IRT_2), soil heat flux (Gs_1, Gs_2, Gs_3), soil moisture (Ms_2cm, Ms_4cm, Ms_10cm, Ms_20cm, Ms_40cm, Ms_60cm, Ms_100cm), soil temperature (Ts_0cm, Ts_2cm, Ts_4cm, Ts_10cm, Ts_20cm, Ts_40cm, Ts_60cm, Ts_100cm). Processing and quality control of observation data: (1) Ensure 144 data per day (every 10 minutes), if there is missing data, it is marked as -6999. (2) Eliminate moments with duplicate records; (3) Remove data that is significantly beyond physical meaning or beyond the measuring range of the instrument. Suspicious data were marked in red. In this dataset, the time of 0:10 corresponds to the average data for the period between 0:00 and 0:10; the data were stored in *.xls format.