Shenshawo station

This dataset includes data recorded by the automatic weather station (AWS) from 2012/06/02 to 2014/12/31. The Shenshawo station (100.49° E, 38.79° N) was covered by sand and located in Shenshawo, eastern Zhangye City, Gansu Province. The elevation is 1594m.

An overview of the measured variables and sensor specifications are summarized in the following Table:

| Variable | Unit | Corporation/Sensors | Height | Direction |
| --- | --- | --- | --- | --- |
| Air temperature | ℃ | Vaisala/HMP45AC | 5,10 m | north |
| Air humidity | % | Vaisala/HMP45AC | 5,10 m | north |
| Wind speed | m/s | RM Young/03001 | 5,10 m | north |
| Wind direction | ° | RM Young/03001 | 5,10 m | north |
| Air pressure | hpa | Vaisala/PTB110 | 2 m | / |
| Precipitation | mm | Texas Electronics/ TE525M | 10 m | / |
| Four-component radiation ((DR/UR/DLR_Cor/ULR_Cor) | W/m2 | Kipp&Zone/CNR1 | 6 m | south |
| Infrared temperature | ℃ | Apogee/SI-111 | 6 m | south, vertically downward |
| Soil heat flux | W/m2 | Hukseflux/HFP01 | -6 cm (3 Repeats) | / |
| Soil temperature | ℃ | Campbell/109 | 0, -2, -4, -10, -20, -40, -60, and -100 cm | / |
| Soil moisture | % | Decagon/ECH2O-5 | -2, -4, -10, -20, -40, -60, and -100 cm | / |

The observed items include: air temperature and humidity (Ta_5m, RH_5m, Ta_10m, RH_10m), pressure(Press), precipitation (Rain), wind speed (WS_5m, WS_10m), wind direction (WD_10m), four-component radiation (DR, UR, DLR_Cor, ULR_Cor, Rn), infrared temperature (IRT_1, IRT_2), soil heat flux (Gs_1, Gs_2, Gs_3), soil moisture (Ms_2cm, Ms_4cm, Ms_10cm, Ms_20cm, Ms_40cm, Ms_60cm, Ms_100cm), soil temperature (Ts_0cm, Ts_2cm, Ts_4cm, Ts_10cm, Ts_20cm, Ts_40cm, Ts_60cm, Ts_100cm). Processing and quality control of observation data: (1) Ensure 144 data per day (every 10 minutes), if there is missing data, it is marked as -6999. (2) Eliminate moments with duplicate records; (3) Remove data that is significantly beyond physical meaning or beyond the measuring range of the instrument. Suspicious data were marked in red. In this dataset, the time of 0:10 corresponds to the average data for the period between 0:00 and 0:10; the data were stored in *.xls format.